Inconvenite Postilion 5.6:

Se evidencia que al momento de realizar transacciones de Pinblock en postilion5.6 arroja error 23 (Invalid PIN block format code. (Use includes where the security setting to implement PCI HSM limitations on PIN Block format usage is applied, and a Host command attempts to convert a PIN Block to a disallowed format.), esto puede ser debido a que el comando JE y JF no estén habilitados en HSM.

Traza hsm:

[Aug 26 13h33:57.226] - Thales RG7000 message received from 172.24.6.186,1500

[None       ans 004 M] : 'Message Header' = [0000]

[None       an  002 M] : 'Response Code' = [ED]

[None       an  002 M] : 'Error Code' = [00]

binary data

0000(0000)  30 30 30 30 45 44 30 30                            0000ED00

[Informational event 149]

[Aug 26 13h33:57.240] - Thales RG7000 message sent to 172.24.6.186,1500

[None       ans 004 M] : 'Message Header' = [0000]

[None       an  002 M] : 'Command Code' = [JE]

[16H/1A+32H/1A+48H/'R'+ans..9999/'S'+ans..9999  M] : 'Source ZPK' = [UCE8BDBB6F3FF5111FB0291452A94E342]

[None       Hex 016 M] : 'PIN Block' = [82268C6C76346437]

[None       n   002 M] : 'PIN Block Format Code' = [01]

[None       n   012 M] : 'Account Number' = [046000003021]

binary data

0000(0000)  30 30 30 30 4a 45 55 43  45 38 42 44 42 42 36 46   0000JEUCE8BDBB6F

0016(0010)  33 46 46 35 31 31 31 46  42 30 32 39 31 34 35 32   3FF5111FB0291452

0032(0020)  41 39 34 45 33 34 32 38  32 32 36 38 43 36 43 37   A94E34282268C6C7

0048(0030)  36 33 34 36 34 33 37 30  31 30 34 36 30 30 30 30   6346437010460000

0064(0040)  30 33 30 32 31                                     03021

[Informational event 148]

[Aug 26 13h33:57.270] - Thales RG7000 message received from 172.24.6.186,1500

[None       ans 004 M] : 'Message Header' = [0000]

[None       an  002 M] : 'Response Code' = [JF]

[None       an  002 M] : 'Error Code' = [23]

binary data

0000(0000)  30 30 30 30 4a 46 32 33                            0000JF23

[Informational event 149]

Traza tm:

[Aug 26 13h33:57.169] - <0600> Message from srNTMFTECH1 [172.20.3.200,58052]

0600:

[LLVAR  n    ..19 016] 002 [4850460000030212]

[Fixed  n       6 006] 003 [920000]

[None   n         012] 004 [000000000000]

[Fixed  n      10 010] 007 [0826133356]

[Fixed  n       6 006] 011 [151748]

[Fixed  n       6 006] 012 [133356]

[Fixed  n       4 004] 013 [0826]

[Fixed  n       4 004] 014 [2403]

[Fixed  n       4 004] 018 [9999]

[Fixed  n       3 003] 022 [010]

[None   n         003] 023 [001]

[Fixed  n       2 002] 025 [00]

[Fixed  n       2 002] 026 [04]

[Fixed  x+n     9 009] 028 [D00000000]

[LLVAR  n    ..11 008] 032 [71300001]

[LLVAR  n    ..11 011] 033 [71300000001]

[Fixed  anp    12 012] 037 [120232232232]

[Fixed  ans     8 008] 041 [99900001]

[Fixed  ans    15 015] 042 [000000201999000]

[Fixed  ans    40 040] 043 [MFTECH                                PA]

[Fixed  a/n     3 003] 049 [840]

[Fixed  b       8 008] 052 *[EA09F5EA81625197]

[Fixed  b      48 048] 053 *[0182268C6C76346437000000000000000000000000000000000000000000000000000000000000000000000000000000]

[LLLVAR ans ..500 025] 059 [NOVOTR_151748120232232232]

[LLLVAR an     15 015] 123 [100000100100119]

[LLVAR  ans  ..32 032] 127.002 [0600:151748:0826133356:73887E405]

[LLLVAR ans ..512 078] 127.040 [12M12320600:151748:0826133356:C0B737A0812M02320600:151748:0826133356:73887E405]

[LLVAR  ans  ..99 017] 127.041 [172.20.1.30,52212]

binary data

0000(0000)  30 36 30 30 f2 3c 46 d1  88 e0 98 20 00 00 00 00   0600.<F.... ....

0016(0010)  00 00 00 22 31 36 34 38  35 30 34 36 30 30 30 30   ..."164850460000

0032(0020)  30 33 30 32 31 32 39 32  30 30 30 30 30 30 30 30   0302129200000000

0048(0030)  30 30 30 30 30 30 30 30  30 38 32 36 31 33 33 33   0000000008261333

0064(0040)  35 36 31 35 31 37 34 38  31 33 33 33 35 36 30 38   5615174813335608

0080(0050)  32 36 32 34 30 33 39 39  39 39 30 31 30 30 30 31   2624039999010001

0096(0060)  30 30 30 34 44 30 30 30  30 30 30 30 30 30 38 37   0004D00000000087

0112(0070)  31 33 30 30 30 30 31 31  31 37 31 33 30 30 30 30   1300001117130000

0128(0080)  30 30 30 31 31 32 30 32  33 32 32 33 32 32 33 32   0001120232232232

0144(0090)  39 39 39 30 30 30 30 31  30 30 30 30 30 30 32 30   9990000100000020

0160(00a0)  31 39 39 39 30 30 30 4d  46 54 45 43 48 20 20 20   1999000MFTECH

0176(00b0)  20 20 20 20 20 20 20 20  20 20 20 20 20 20 20 20

0192(00c0)  20 20 20 20 20 20 20 20  20 20 20 20 20 50 41 38                PA8

0208(00d0)  34 30 ea 09 f5 ea 81 62  51 97 01 82 26 8c 6c 76   40.....bQ...&.lv

0224(00e0)  34 64 37 00 00 00 00 00  00 00 00 00 00 00 00 00   4d7.............

0240(00f0)  00 00 00 00 00 00 00 00  00 00 00 00 00 00 00 00   ................

0256(0100)  00 00 00 00 00 00 00 00  00 00 30 32 35 4e 4f 56   ..........025NOV

0272(0110)  4f 54 52 5f 31 35 31 37  34 38 31 32 30 32 33 32   OTR_151748120232

0288(0120)  32 33 32 32 33 32 30 31  35 31 30 30 30 30 30 31   2322320151000001

0304(0130)  30 30 31 30 30 31 31 39  30 30 30 31 34 32 40 00   00100119000142@.

0320(0140)  00 00 01 80 00 00 33 32  30 36 30 30 3a 31 35 31   ......320600:151

0336(0150)  37 34 38 3a 30 38 32 36  31 33 33 33 35 36 3a 37   748:0826133356:7

0352(0160)  33 38 38 37 45 34 30 35  30 37 38 31 32 4d 31 32   3887E40507812M12

0368(0170)  33 32 30 36 30 30 3a 31  35 31 37 34 38 3a 30 38   320600:151748:08

0384(0180)  32 36 31 33 33 33 35 36  3a 43 30 42 37 33 37 41   26133356:C0B737A

0400(0190)  30 38 31 32 4d 30 32 33  32 30 36 30 30 3a 31 35   0812M02320600:15

0416(01a0)  31 37 34 38 3a 30 38 32  36 31 33 33 33 35 36 3a   1748:0826133356:

0432(01b0)  37 33 38 38 37 45 34 30  35 31 37 31 37 32 2e 32   73887E40517172.2

0448(01c0)  30 2e 31 2e 33 30 2c 35  32 32 31 32               0.1.30,52212

[Aug 26 13h33:57.270] - Error event 140

An implementation specific error occurred in the Thales RG7000 cryptographic implementation. The error was: The HSM specific PIN block format code was invalid.

stack trace

Postilion exception: [postilion.realtime.sdk.crypto.XCryptoImplError]

Description: An implementation specific error occurred in the Thales RG7000 cryptographic implementation. The error was: The HSM specific PIN block format code was invalid.

ID: [140]

Data: [none]

at postilion.realtime.sdk.crypto.impl.rg7000.ARG7000KeyImpl.processErrorCode(ARG7000KeyImpl.java:160)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKeyImpl.processErrorCode(RG7000DesKeyImpl.java:1875)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKeyImpl.processErrorCode(RG7000DesKeyImpl.java:1821)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKeyImpl.processResponseAndErrorCode(RG7000DesKeyImpl.java:1905)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKvpImpl.translatePinToLmk(RG7000DesKvpImpl.java:727)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKvpVisaImpl.generatePvv(RG7000DesKvpVisaImpl.java:370)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKvpVisaImpl.generatePvv(RG7000DesKvpVisaImpl.java:300)

at postilion.realtime.sdk.crypto.DesKvpVisa.generatePvv(DesKvpVisa.java:280)

at postilion.realtime.sdk.crypto.DesKvpVisaGroup.generatePvv(DesKvpVisaGroup.java:285)

at postilion.postcard.authorizers.VISAPinVerificationData.changePin(VISAPinVerificationData.java:332)

at postilion.postcard.authorizers.AuthorizerPostCard.authorizePINChangeRequest(AuthorizerPostCard.java:4838)

at postilion.postcard.authorizers.AuthorizerPostCard.authorizeAdminRequest(AuthorizerPostCard.java:4516)

at postilion.postcard.authorizers.AuthorizerPostCard.authorize(AuthorizerPostCard.java:3118)

at postilion.postcard.authorizers.AuthorizerPostCard.authorizeRequestOnline(AuthorizerPostCard.java:579)

at postilion.realtime.apps.tranmgr.EventHandlerReqReqMessage.attemptLocalAuthorization(EventHandlerReqReqMessage.java:149)

at postilion.realtime.apps.tranmgr.EventHandlerAdminReq.authorizeOrSendOnline(EventHandlerAdminReq.java:114)

at postilion.realtime.apps.tranmgr.EventHandlerAdminReq.processTran(EventHandlerAdminReq.java:85)

at postilion.realtime.apps.tranmgr.EventHandlerMessage.process(EventHandlerMessage.java:60)

at postilion.realtime.apps.tranmgr.EventHandlerMessage.processMsg(EventHandlerMessage.java:37)

at postilion.realtime.apps.tranmgr.TransactionManager.processNodeMessage(TransactionManager.java:1971)

at postilion.realtime.apps.tranmgr.TransactionManager.processEvent(TransactionManager.java:1843)

at postilion.realtime.sdk.util.Processor.run(Processor.java:214)

at postilion.realtime.sdk.env.AppProcessor.run(AppProcessor.java:157)

[Error event 140]

[Aug 26 13h33:57.272] - Warning event 5127

Message from [srNTMFTECH1, skMFTECH] (Transaction number: 1956843, PAN or track 2: 4850460000030212, Message type: 0600, Source node: srNTMFTECH1, Sink node: skMFTECH, Acquirer ID: 71300001, Card acceptor ID: [unknown], Terminal ID or name: [unknown]) declined because of a system malfunction or similar error. A response code of 96 was returned for transaction with PAN or track 2 4850460000030212 and Card Sequence Number 001.

stack trace

Postilion exception: [postilion.realtime.apps.tranmgr.XTmDeclineTran]

Description: Message from [srNTMFTECH1, skMFTECH] (Transaction number: 1956843, PAN or track 2: 485046******0212, Message type: 0600, Source node: srNTMFTECH1, Sink node: skMFTECH, Acquirer ID: 71300001, Card acceptor ID: [unknown], Terminal ID or name: [unknown]) declined because of a system malfunction or similar error. A response code of 96 was returned for transaction with PAN or track 2 485046******0212 and Card Sequence Number 001.

ID: [5127]

Data: [none]

at postilion.postcard.authorizers.AuthorizerPostCard.authorizeRequestOnline(AuthorizerPostCard.java:660)

at postilion.realtime.apps.tranmgr.EventHandlerReqReqMessage.attemptLocalAuthorization(EventHandlerReqReqMessage.java:149)

at postilion.realtime.apps.tranmgr.EventHandlerAdminReq.authorizeOrSendOnline(EventHandlerAdminReq.java:114)

at postilion.realtime.apps.tranmgr.EventHandlerAdminReq.processTran(EventHandlerAdminReq.java:85)

at postilion.realtime.apps.tranmgr.EventHandlerMessage.process(EventHandlerMessage.java:60)

at postilion.realtime.apps.tranmgr.EventHandlerMessage.processMsg(EventHandlerMessage.java:37)

at postilion.realtime.apps.tranmgr.TransactionManager.processNodeMessage(TransactionManager.java:1971)

at postilion.realtime.apps.tranmgr.TransactionManager.processEvent(TransactionManager.java:1843)

at postilion.realtime.sdk.util.Processor.run(Processor.java:214)

at postilion.realtime.sdk.env.AppProcessor.run(AppProcessor.java:157)

Caused by: Postilion exception: [postilion.realtime.sdk.crypto.XCryptoImplError]

Description: An implementation specific error occurred in the Thales RG7000 cryptographic implementation. The error was: The HSM specific PIN block format code was invalid.

ID: [140]

Data: [none]

at postilion.realtime.sdk.crypto.impl.rg7000.ARG7000KeyImpl.processErrorCode(ARG7000KeyImpl.java:160)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKeyImpl.processErrorCode(RG7000DesKeyImpl.java:1875)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKeyImpl.processErrorCode(RG7000DesKeyImpl.java:1821)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKeyImpl.processResponseAndErrorCode(RG7000DesKeyImpl.java:1905)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKvpImpl.translatePinToLmk(RG7000DesKvpImpl.java:727)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKvpVisaImpl.generatePvv(RG7000DesKvpVisaImpl.java:370)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKvpVisaImpl.generatePvv(RG7000DesKvpVisaImpl.java:300)

at postilion.realtime.sdk.crypto.DesKvpVisa.generatePvv(DesKvpVisa.java:280)

at postilion.realtime.sdk.crypto.DesKvpVisaGroup.generatePvv(DesKvpVisaGroup.java:285)

at postilion.postcard.authorizers.VISAPinVerificationData.changePin(VISAPinVerificationData.java:332)

at postilion.postcard.authorizers.AuthorizerPostCard.authorizePINChangeRequest(AuthorizerPostCard.java:4838)

at postilion.postcard.authorizers.AuthorizerPostCard.authorizeAdminRequest(AuthorizerPostCard.java:4516)

at postilion.postcard.authorizers.AuthorizerPostCard.authorize(AuthorizerPostCard.java:3118)

at postilion.postcard.authorizers.AuthorizerPostCard.authorizeRequestOnline(AuthorizerPostCard.java:579)

... 9 more

[Warning event 5127]

ERROR TRANSACCIONES E-COMERCE POSTILION 5.3:

Se evidencia que al momento de realizar  una compra de E-comerce la transacción falla debido a que el comando MQ se responde con el MR con error 67 (Command not licensed), esto puede ser debido a que el comando no este habilitado, puesto a que al hacer el rollback de el apuntamiento del HSM la transacción es contestada satisfactoriamente.

TRACE HSM:

Aug 25 11h19:05.695] - Thales RG7000 message sent to 172.24.6.186,1500

[None       ans 004 M] : 'Message Header' = [0000]

[None       an  002 M] : 'Command Code' = [MQ]

[None       n   001 M] : 'Message Block Number' = [0]

[16H/1A+32H/1A+48H  M] : 'ZAK' = 'U00A64E97D4EBDA4DCED699B0341D2F4A'

[None       Hex 003 M] : 'Message Length' = [180]

[0..4096    [^0x3,0x19]096 M] : 'Message Block' = [ISO0260000000200B238800128E1801A000000001400000535001000000000400008251619040290091119050825030506400551378810170513015248977=6012888000000000012371602900900000000TERMID01100000160848702ACQUIRER NAME         CITY NAME       US027                   0100    840016NOVONOVO+00000000164896350000000131008TRADCT011100000000000280000000000000000000000000000030000000000000000000000000000000]

binary data

0000(0000)  30 30 30 30 4d 51 30 55  30 30 41 36 34 45 39 37   0000MQ0U00A64E97

0016(0010)  44 34 45 42 44 41 34 44  43 45 44 36 39 39 42 30   D4EBDA4DCED699B0

0032(0020)  33 34 31 44 32 46 34 41  31 38 30 49 53 4f 30 32   341D2F4A180ISO02

0048(0030)  36 30 30 30 30 30 30 30  32 30 30 42 32 33 38 38   60000000200B2388

0064(0040)  30 30 31 32 38 45 31 38  30 31 41 30 30 30 30 30   00128E1801A00000

0080(0050)  30 30 30 31 34 30 30 30  30 30 35 33 35 30 30 31   0001400000535001

0096(0060)  30 30 30 30 30 30 30 30  30 34 30 30 30 30 38 32   0000000004000082

0112(0070)  35 31 36 31 39 30 34 30  32 39 30 30 39 31 31 31   5161904029009111

0128(0080)  39 30 35 30 38 32 35 30  33 30 35 30 36 34 30 30   9050825030506400

0144(0090)  35 35 31 33 37 38 38 31  30 31 37 30 35 31 33 30   5513788101705130

0160(00a0)  31 35 32 34 38 39 37 37  3d 36 30 31 32 38 38 38   15248977=6012888

0176(00b0)  30 30 30 30 30 30 30 30  30 30 31 32 33 37 31 36   0000000000123716

0192(00c0)  30 32 39 30 30 39 30 30  30 30 30 30 30 30 54 45   02900900000000TE

0208(00d0)  52 4d 49 44 30 31 31 30  30 30 30 30 31 36 30 38   RMID011000001608

0224(00e0)  34 38 37 30 32 41 43 51  55 49 52 45 52 20 4e 41   48702ACQUIRER NA

0240(00f0)  4d 45 20 20 20 20 20 20  20 20 20 43 49 54 59 20   ME         CITY

0256(0100)  4e 41 4d 45 20 20 20 20  20 20 20 55 53 30 32 37   NAME       US027

0272(0110)  20 20 20 20 20 20 20 20  20 20 20 20 20 20 20 20

0288(0120)  20 20 20 30 31 30 30 20  20 20 20 38 34 30 30 31      0100    84001

0304(0130)  36 4e 4f 56 4f 4e 4f 56  4f 2b 30 30 30 30 30 30   6NOVONOVO+000000

0320(0140)  30 30 31 36 34 38 39 36  33 35 30 30 30 30 30 30   0016489635000000

0336(0150)  30 31 33 31 30 30 38 54  52 41 44 43 54 30 31 31   0131008TRADCT011

0352(0160)  31 30 30 30 30 30 30 30  30 30 30 30 32 38 30 30   1000000000002800

0368(0170)  30 30 30 30 30 30 30 30  30 30 30 30 30 30 30 30   0000000000000000

0384(0180)  30 30 30 30 30 30 30 30  30 30 30 33 30 30 30 30   0000000000030000

0400(0190)  30 30 30 30 30 30 30 30  30 30 30 30 30 30 30 30   0000000000000000

0416(01a0)  30 30 30 30 30 30 30 30  30 30 30                  00000000000

--------------------------------------------------------------------------------

[Aug 25 11h19:05.708] - Thales RG7000 message received from 172.24.6.186,1500

[None       ans 004 M] : 'Message Header' = [0000]

[None       an  002 M] : 'Response Code' = [MR]

[None       an  002 M] : 'Error Code' = [67]

binary data

0000(0000)  30 30 30 30 4d 52 36 37                            0000MR67

TRACE HSM SINK:

[Fixed  n       4 004] 016 [0825]

[Fixed  n       4 004] 018 [5972]

[Fixed  n       3 003] 022 [012]

[Fixed  n       2 002] 025 [08]

[Fixed  x+n     9 009] 028 [C00000000]

[Fixed  x+n     9 009] 029 [C00000000]

[Fixed  x+n     9 009] 030 [C00000000]

[Fixed  x+n     9 009] 031 [C00000000]

[LLVAR  n    ..11 006] 032 [400551]

[Fixed  anp    12 012] 037 [123716029009]

[Fixed  ans     8 008] 041 [TERMID01]

[Fixed  ans    15 015] 042 [CARD ACCEPTOR  ]

[Fixed  ans    40 040] 043 [ACQUIRER NAME          CITY NAME      US]

[Fixed  a/n     3 003] 049 [840]

[Fixed  a/n     3 003] 050 [170]

[LLLVAR n       4 004] 056 [1510]

[LLVAR  ans  ..28 010] 102 [3015248977]

[LLLVAR an     15 015] 123 [000020100000190]

[LLVAR  ans  ..32 010] 127.002 [0004220383]

[Fixed  ans*   48 048] 127.003 [srVISANETSIMskRappiPayCO029009029009tgRPPCOL    ]

[Fixed  ans*   17 017] 127.013 [              840]

[Fixed  n       8 008] 127.020 [20210305]

[LLLLLVAans 99999 385] 127.022 [234ONLINE_RISK_ASSESSMENT_REASON_CODE14095A218Postilion:MetaData3162234ONLINE_RISK_ASSESSMENT_REASON_CODE111237ONLINE_RISK_ASSESSMENT_CONDITION_CODE111227MERCHANT_VERIFICATION_VALUE111214TRANSACTION_ID111220OriginalPosEntryMode111237ONLINE_RISK_ASSESSMENT_CONDITION_CODE1602C200227MERCHANT_VERIFICATION_VALUE2100000530000214TRANSACTION_ID215301237587457487220OriginalPosEntryMode140120]

[LLVAR  ans  ..11 004] 127.035 [0002]

[LLVAR  ans  ..25 010] 127.036 [1160848702]

[LLLVAR ans ..512 039] 127.040 [12M02327911BB5A1A7B60071A4B1AE059D8D71C]

binary data

0000(0000)  30 31 30 30 fa bf 44 9f  08 e0 c1 00 00 00 00 00   0100..D.........

0016(0010)  04 00 00 22 31 36 34 38  39 36 33 35 30 30 30 30   ..."164896350000

0032(0020)  30 30 30 31 33 31 30 30  31 30 30 30 30 30 30 30   0001310010000000

0048(0030)  30 30 30 30 34 30 30 30  30 30 30 30 31 35 36 32   0000400000001562

0064(0040)  35 36 30 34 30 38 32 35  31 36 31 39 30 34 33 33   5604082516190433

0080(0050)  39 30 36 34 30 31 30 32  39 30 30 39 31 31 31 39   9064010290091119

0096(0060)  30 35 30 38 32 35 32 34  30 33 30 33 30 35 30 38   0508252403030508

0112(0070)  32 35 35 39 37 32 30 31  32 30 38 43 30 30 30 30   25597201208C0000

0128(0080)  30 30 30 30 43 30 30 30  30 30 30 30 30 43 30 30   0000C00000000C00

0144(0090)  30 30 30 30 30 30 43 30  30 30 30 30 30 30 30 30   000000C000000000

0160(00a0)  36 34 30 30 35 35 31 31  32 33 37 31 36 30 32 39   6400551123716029

0176(00b0)  30 30 39 54 45 52 4d 49  44 30 31 43 41 52 44 20   009TERMID01CARD

0192(00c0)  41 43 43 45 50 54 4f 52  20 20 41 43 51 55 49 52   ACCEPTOR  ACQUIR

0208(00d0)  45 52 20 4e 41 4d 45 20  20 20 20 20 20 20 20 20   ER NAME

0224(00e0)  20 43 49 54 59 20 4e 41  4d 45 20 20 20 20 20 20    CITY NAME

0240(00f0)  55 53 38 34 30 31 37 30  30 30 34 31 35 31 30 31   US84017000415101

0256(0100)  30 33 30 31 35 32 34 38  39 37 37 30 31 35 30 30   0301524897701500

0272(0110)  30 30 32 30 31 30 30 30  30 30 31 39 30 30 30 30   0020100000190000

0288(0120)  35 34 33 60 08 14 00 31  00 00 00 31 30 30 30 30   543`...1...10000

0304(0130)  34 32 32 30 33 38 33 73  72 56 49 53 41 4e 45 54   4220383srVISANET

0320(0140)  53 49 4d 73 6b 52 61 70  70 69 50 61 79 43 4f 30   SIMskRappiPayCO0

0336(0150)  32 39 30 30 39 30 32 39  30 30 39 74 67 52 50 50   29009029009tgRPP

0352(0160)  43 4f 4c 20 20 20 20 20  20 20 20 20 20 20 20 20   COL

0368(0170)  20 20 20 20 20 38 34 30  32 30 32 31 30 33 30 35        84020210305

0384(0180)  30 30 33 38 35 32 33 34  4f 4e 4c 49 4e 45 5f 52   00385234ONLINE_R

0400(0190)  49 53 4b 5f 41 53 53 45  53 53 4d 45 4e 54 5f 52   ISK_ASSESSMENT_R

0416(01a0)  45 41 53 4f 4e 5f 43 4f  44 45 31 34 30 39 35 41   EASON_CODE14095A

0432(01b0)  32 31 38 50 6f 73 74 69  6c 69 6f 6e 3a 4d 65 74   218Postilion:Met

0448(01c0)  61 44 61 74 61 33 31 36  32 32 33 34 4f 4e 4c 49   aData3162234ONLI

0464(01d0)  4e 45 5f 52 49 53 4b 5f  41 53 53 45 53 53 4d 45   NE_RISK_ASSESSME

0480(01e0)  4e 54 5f 52 45 41 53 4f  4e 5f 43 4f 44 45 31 31   NT_REASON_CODE11

0496(01f0)  31 32 33 37 4f 4e 4c 49  4e 45 5f 52 49 53 4b 5f   1237ONLINE_RISK_

0512(0200)  41 53 53 45 53 53 4d 45  4e 54 5f 43 4f 4e 44 49   ASSESSMENT_CONDI

0528(0210)  54 49 4f 4e 5f 43 4f 44  45 31 31 31 32 32 37 4d   TION_CODE111227M

0544(0220)  45 52 43 48 41 4e 54 5f  56 45 52 49 46 49 43 41   ERCHANT_VERIFICA

0560(0230)  54 49 4f 4e 5f 56 41 4c  55 45 31 31 31 32 31 34   TION_VALUE111214

0576(0240)  54 52 41 4e 53 41 43 54  49 4f 4e 5f 49 44 31 31   TRANSACTION_ID11

0592(0250)  31 32 32 30 4f 72 69 67  69 6e 61 6c 50 6f 73 45   1220OriginalPosE

0608(0260)  6e 74 72 79 4d 6f 64 65  31 31 31 32 33 37 4f 4e   ntryMode111237ON

0624(0270)  4c 49 4e 45 5f 52 49 53  4b 5f 41 53 53 45 53 53   LINE_RISK_ASSESS

0640(0280)  4d 45 4e 54 5f 43 4f 4e  44 49 54 49 4f 4e 5f 43   MENT_CONDITION_C

0656(0290)  4f 44 45 31 36 30 32 43  32 30 30 32 32 37 4d 45   ODE1602C200227ME

0672(02a0)  52 43 48 41 4e 54 5f 56  45 52 49 46 49 43 41 54   RCHANT_VERIFICAT

0688(02b0)  49 4f 4e 5f 56 41 4c 55  45 32 31 30 30 30 30 30   ION_VALUE2100000

0704(02c0)  35 33 30 30 30 30 32 31  34 54 52 41 4e 53 41 43   530000214TRANSAC

0720(02d0)  54 49 4f 4e 5f 49 44 32  31 35 33 30 31 32 33 37   TION_ID215301237

0736(02e0)  35 38 37 34 35 37 34 38  37 32 32 30 4f 72 69 67   587457487220Orig

0752(02f0)  69 6e 61 6c 50 6f 73 45  6e 74 72 79 4d 6f 64 65   inalPosEntryMode

0768(0300)  31 34 30 31 32 30 30 34  30 30 30 32 31 30 31 31   1401200400021011

0784(0310)  36 30 38 34 38 37 30 32  30 33 39 31 32 4d 30 32   6084870203912M02

0800(0320)  33 32 37 39 31 31 42 42  35 41 31 41 37 42 36 30   327911BB5A1A7B60

0816(0330)  30 37 31 41 34 42 31 41  45 30 35 39 44 38 44 37   071A4B1AE059D8D7

0832(0340)  31 43                                              1C

--------------------------------------------------------------------------------

[Aug 25 11h19:05.708] - Error event 140

An implementation specific error occurred in the Thales RG7000 cryptographic implementation. The error was: The HSM responded to request with the error code: 67. The command code in the request was MQ.

stack trace

Postilion exception: [postilion.realtime.sdk.crypto.XCryptoImplError]

Description: An implementation specific error occurred in the Thales RG7000 cryptographic implementation. The error was: The HSM responded to request with the error code: 67. The command code in the request was MQ.

ID: [140]

Data: [none]

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKeyImpl.processErrorCode(RG7000DesKeyImpl.java:1430)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKeyImpl.processErrorCode(RG7000DesKeyImpl.java:1289)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKeyImpl.processResponseAndErrorCode(RG7000DesKeyImpl.java:1481)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKwaImpl.authenticateWithZak(RG7000DesKwaImpl.java:389)

at postilion.realtime.sdk.crypto.impl.rg7000.RG7000DesKwaImpl.authenticate(RG7000DesKwaImpl.java:189)

at postilion.realtime.sdk.crypto.DesKwa.authenticate(DesKwa.java:134)

at postilion.realtime.base24.MACField.constructMAC(MACField.java:114)

at postilion.realtime.base24.Base24Bem.constructMACToRemote(Base24Bem.java:10111)

at postilion.realtime.base24.Base24DaviPay.processMsgRequest(Base24DaviPay.java:122)

at postilion.realtime.base24.Base24DaviPay.processAuthReqFromTranmgr(Base24DaviPay.java:178)

at postilion.realtime.sdk.node.AInterchangeDriver.processMsgFromTranmgr(AInterchangeDriver.java:1119)

at postilion.realtime.sdk.node.ProfilingInterchangeDriver.processMsgFromTranmgr(ProfilingInterchangeDriver.java:197)

at postilion.realtime.sdk.node.Interchange.processMsgFromTranmgr(Interchange.java:4085)

at postilion.realtime.sdk.node.Interchange.processDataEventFromTranmgr(Interchange.java:3954)

at postilion.realtime.sdk.node.Interchange.processEndpointData(Interchange.java:686)

at postilion.realtime.sdk.node.InterchangeProcessor.processEndpointData(InterchangeProcessor.java:798)

at postilion.realtime.sdk.node.InterchangeProcessor.processEvent(InterchangeProcessor.java:537)

at postilion.realtime.sdk.util.Processor.run(Processor.java:213)

at postilion.realtime.sdk.env.AppProcessor.run(AppProcessor.java:130)

TRACE HSM ROLLBACK:

Aug 25 12h19:55.933] - Thales RG7000 message sent to 172.17.37.155,1500

[None       ans 004 M] : 'Message Header' = [0000]

[None       an  002 M] : 'Command Code' = [MQ]

[None       n   001 M] : 'Message Block Number' = [0]

[16H/1A+32H/1A+48H  M] : 'ZAK' = 'U00A64E97D4EBDA4DCED699B0341D2F4A'

[None       Hex 003 M] : 'Message Length' = [180]

[0..4096    [^0x3,0x19]096 M] : 'Message Block' = [ISO0260000000200B238800128E1801A000000001400000535001000000000100008251719470290111219480825030506400552378810170513015248977=6012888000000000012371702901100000000TERMID01100000160848702ACQUIRER NAME         CITY NAME       US027                   0100    840016NOVONOVO+00000000164896350000000131008TRADCT011100000000000280000000000000000000000000000030000000000000000000000000000000]

binary data

0000(0000)  30 30 30 30 4d 51 30 55  30 30 41 36 34 45 39 37   0000MQ0U00A64E97

0016(0010)  44 34 45 42 44 41 34 44  43 45 44 36 39 39 42 30   D4EBDA4DCED699B0

0032(0020)  33 34 31 44 32 46 34 41  31 38 30 49 53 4f 30 32   341D2F4A180ISO02

0048(0030)  36 30 30 30 30 30 30 30  32 30 30 42 32 33 38 38   60000000200B2388

0064(0040)  30 30 31 32 38 45 31 38  30 31 41 30 30 30 30 30   00128E1801A00000

0080(0050)  30 30 30 31 34 30 30 30  30 30 35 33 35 30 30 31   0001400000535001

0096(0060)  30 30 30 30 30 30 30 30  30 31 30 30 30 30 38 32   0000000001000082

0112(0070)  35 31 37 31 39 34 37 30  32 39 30 31 31 31 32 31   5171947029011121

0128(0080)  39 34 38 30 38 32 35 30  33 30 35 30 36 34 30 30   9480825030506400

0144(0090)  35 35 32 33 37 38 38 31  30 31 37 30 35 31 33 30   5523788101705130

0160(00a0)  31 35 32 34 38 39 37 37  3d 36 30 31 32 38 38 38   15248977=6012888

0176(00b0)  30 30 30 30 30 30 30 30  30 30 31 32 33 37 31 37   0000000000123717

0192(00c0)  30 32 39 30 31 31 30 30  30 30 30 30 30 30 54 45   02901100000000TE

0208(00d0)  52 4d 49 44 30 31 31 30  30 30 30 30 31 36 30 38   RMID011000001608

0224(00e0)  34 38 37 30 32 41 43 51  55 49 52 45 52 20 4e 41   48702ACQUIRER NA

0240(00f0)  4d 45 20 20 20 20 20 20  20 20 20 43 49 54 59 20   ME         CITY

0256(0100)  4e 41 4d 45 20 20 20 20  20 20 20 55 53 30 32 37   NAME       US027

0272(0110)  20 20 20 20 20 20 20 20  20 20 20 20 20 20 20 20

0288(0120)  20 20 20 30 31 30 30 20  20 20 20 38 34 30 30 31      0100    84001

0304(0130)  36 4e 4f 56 4f 4e 4f 56  4f 2b 30 30 30 30 30 30   6NOVONOVO+000000

0320(0140)  30 30 31 36 34 38 39 36  33 35 30 30 30 30 30 30   0016489635000000

0336(0150)  30 31 33 31 30 30 38 54  52 41 44 43 54 30 31 31   0131008TRADCT011

0352(0160)  31 30 30 30 30 30 30 30  30 30 30 30 32 38 30 30   1000000000002800

0368(0170)  30 30 30 30 30 30 30 30  30 30 30 30 30 30 30 30   0000000000000000

0384(0180)  30 30 30 30 30 30 30 30  30 30 30 33 30 30 30 30   0000000000030000

0400(0190)  30 30 30 30 30 30 30 30  30 30 30 30 30 30 30 30   0000000000000000

0416(01a0)  30 30 30 30 30 30 30 30  30 30 30                  00000000000

--------------------------------------------------------------------------------

[Aug 25 12h19:55.948] - Thales RG7000 message received from 172.17.37.155,1500

[None       ans 004 M] : 'Message Header' = [0000]

[None       an  002 M] : 'Response Code' = [MR]

[None       an  002 M] : 'Error Code' = [00]

[None       Hex 016 M] : 'MAB' = [7AAA88897E2348DD]

binary data

0000(0000)  30 30 30 30 4d 52 30 30  37 41 41 41 38 38 38 39   0000MR007AAA8889

0016(0010)  37 45 32 33 34 38 44 44                            7E2348DD